|  | Simulation of GEMINI-01-01-01 Date: Thursday, January 8, 2026 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Interaction Information 5 Mesh information 6 Sensor Details 7 Resultant Forces 8 Beams 8 Study Results 9 Conclusion 12. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: GEMINI-01-01-01 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line1; Solid Body; Mass:0.082941 kg Volume:3.07189e-05 m^3 Density:2,700 kg/m^3 Weight:0.812822 N; C:\Users\williamli\Desktop\GEMINI\CAD\GEMINI-01-01-01.SLDPRT Jan 8 11:35:21 2026.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Contact penalty stiffness scale factor; 1. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\williamli\Desktop\GEMINI\CAD).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.75e+08 N/m^2. Tensile strength:; 3.1e+08 N/m^2. Elastic modulus:; 6.9e+10 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2,700 kg/m^3. Shear modulus:; 2.6e+10 N/m^2. Thermal expansion coefficient:; 2.4e-05 /Kelvin.; SolidBody 1(#31 (Split Line1)(GEMINI-01-01-01). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 5 edge(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 8.34465e-07; 0.962499; -8.49366e-07; 0.962499. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 4 face(s). Type:; Apply normal force. Value:; -0.9625 N..

# Connector Definitions No Data

# Interaction Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 0.246688 in. Minimum element size; 0.0376657 in. Mesh Quality; High. ## Mesh information - Details Total Nodes; 14668. Total Elements; 7500. Maximum Aspect Ratio; 6.7954. % of elements with Aspect Ratio < 3; 98.4. Percentage of elements with Aspect Ratio > 10; 0. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:03. Computer name:; WILLIAMLI2E83. ## Mesh Quality Plots Name; Type; Min; Max. Quality1; Mesh; -; -. GEMINI-01-01-01-Static 1-Quality-Quality1.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 8.34465e-07; 0.962499; -8.49366e-07; 0.962499. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 8.6776e-07; -1.04774e-07; -8.49832e-07; 1.2191e-06. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 1.065e-05N/mm^2 (MPa) Node: 2568; 4.395e+00N/mm^2 (MPa) Node: 419. GEMINI-01-01-01-Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mil Node: 35; 3.445e-01mil Node: 1058. GEMINI-01-01-01-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 8.163e-11 Element: 5773; 2.375e-05 Element: 3975. GEMINI-01-01-01-Static 1-Strain-Strain1. Image-1.

# Conclusion