GEMINI: Graphene-Foil Electron-Emission Mapping Investigation for Novel Instrumentation

In space physics, amorphous carbon foils are a standard tool in particle-detecting space instruments because they release secondary electrons when struck by an incident particle. These secondary electrons can be used to make coincidence or time-of-flight (TOF) measurements, which are used by scientists to deduce properties of the incident particle such as mass, species, and energy. However, little is known beyond a handful of published experiments, about exactly how the process of secondary electron emission works and how foil thickness and composition affect the fluence of secondary electrons.

To address this gap, this proposal aims to develop an ultra-high-vacuum (UHV) compatible scientific instrument that holds a carbon foil in the path of a particle beam and partitions particles passing through the foil based on charge state while measuring the fluence of secondary electrons emitted from the carbon foil as it interacts with the beam. The instrument will be designed based on an existing device built by students in the 2023-2024 AST250/251 class which was used to image carbon foils in the path of an ion beam and quantify the effect of holes and wrinkles (defects) in the foil on its performance. The new instrument created as part of this research will hold at least three carbon-based foils, concentrically partition charges, and measure secondary electron emission from the foils allowing the instrument operator to measure the effect of different types of foils, such as amorphous carbon foils and few-layer-graphene foils (FLGs) under the same particle beam. The end scientific goal of this research is to create an empirical model that infers the thickness and composition of the measured carbon-based foil based on the fluence of secondary electrons emitted from the foil.

In current literature it is hypothesized that the thinner the foil, the more sensitive a space instrument will be to detecting higher mass particles and lower energy particles, so a graphene foil, as a single layer of carbon, should perform significantly better than amorphous carbon. However, existing experimentally-based findings in the literature does not support that theory and shows that graphene only has marginal improvements over amorphous carbon. It is hypothesized that there is a “crud layer” of water vapor and other trace elements that accumulates on carbon-based foils which significantly increases the thickness and erases the supposed benefits of a thinner foil. In a laboratory setting, future researchers can use GEMINI to test different graphene foils with varying thicknesses and surface compositions to search for the optimal graphene foil for space flight. The development of GEMINI holds tremendous scientific value. It will mark the first ever study of secondary electron emission as a function of graphene foil thickness and composition and thus will inform the qualification of graphene foils as the next generation of carbon foils in space instruments.

As previously mentioned, this project will build upon an apparatus first designed by the 2023-2024 AST250/251 class in the Space Physics Lab. The class’s instrument uses three linear actuators to move a pinhole and carbon foil in front of a microchannel plate (MCP) imager so that when the device is placed in line with a beam of particles in the Space Physics Lab’s vacuum chamber, the MCP creates a 2D spatial map of particle counts. It also has a metal gate (consisting of 2 parallel rods) that can be energized to high voltage and serves as an electrostatic analyzer (ESA), deflecting particles based on their energy and charge. This creates a visual distribution on the MCP which can be used to quantify how many of the particles emitted from the foil came off as negatively charged, positively charged, or neutral, as well as how much charge they carried (+1, +2, etc.). The ESA gate can also filter out particles to only look at one specific energy at a time.

There are some flaws with the AST250/251 design that primarily relate to the actuation method, which cannot precisely control the position of the pinhole and carbon foils as intended and is prone to getting stuck, making the device unreliable. The entire device is also tilted at a 45-degree angle to facilitate retraction of the linear actuators which are only controllable in the extending direction, which poses challenges for experimental reproducibility and also needs to be re-examined. GEMINI will improve upon the AST250/251 instrument mechanically by implementing a different actuation mechanism, possibly a rotating wheel rather than linear actuators. A new concentric ESA will also be designed to replace the existing metal gate ESA because the metal gate design shapes the paths of ions in a rectangular pattern, resulting in lost data when the particles collide with the MCP which has a circular aperture. Building upon the AST250/251 design, to meet the goals of this research, a channel electron multiplier (CEM) and new ion optics will be added to the design to take secondary electron measurements. CEMs multiply 1 electron to ~107-108 electrons, essentially creating a cascade of electrons with enough charge to be turned into a detectable signal.

For this project, SolidWorks CAD software will be used to mechanically design GEMINI and files from SolidWorks will be exported to Ansys for analysis and SIMION for electro-optical simulations. From an educational development perspective, this independent research is an opportunity to improve CAD skills, specifically in Design for Manufacturing (DFM). This project is also an opportunity to learn how to perform simulations in Ansys, including, but not limited to structural FEA and thermal simulations. Lastly, the project will also provide experience with SIMION, specifically in verifying simulation results by comparing them to collected data.

In total, the project duration spans both the Fall 2025 and Spring 2026 semesters. For Fall 2025 the requirements for this project are to (1) complete the redesign of the existing AST250/251 class instrument, (2) achieve a 15% reduction in mass without increasing the volume of the instrument, and (3) test the instrument and achieve a reliability of 10 cumulative hours in a beamline without a fault. In Spring 2026, with the instrument built, the focus of the research will shift to taking scientific measurements of amorphous carbon foils and FLGs and creating the model that connects the foil’s thickness and composition properties to the data collected by the instrument. The foils will then be taken to Princeton’s Imaging and Analysis Center (IAC) for foil thickness and composition measurements, in order to quantify the agreement between the model and IAC measurements, with a goal of achieving agreement to within 10%.

In terms of project management, this research involves a weekly advisor-student meeting to ensure continuous satisfactory progress, attendance at the Space Physics Lab weekly standup at 8AM on the first day of the week, and scheduled time to work in the lab from 1PM-5PM on Thursdays and 8:30AM-5PM on Fridays.